| - Identification of the substance/mixture and the company/undertaking |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ## 1.1 Product identifier |  |  |  |  |  |
| Product name |  |  | ## Soap composition "Seam Shine" for forming seams of epoxy grouts and sealants TM "DipoxSi Systems" ("Dipoxi Systems") |  |  |
| ## 1.2 Relevant identified uses of the substance or mixture and uses advised against |  |  |  |  |  |
| Brief recommendations for use |  |  | This soap-based compound is designed for wet finishing (smoothing) of joints filled with one- or two-component epoxy grout, as well as any type of sealant (silicone, polyurethane, acrylic, etc.) from DipoxSi Systems. The product ensures a smooth, even joint surface without damaging the material and significantly facilitates the removal of excess from adjacent surfaces. |  |  |
| 1.3 Manufacturer Information |  |  |  |  |  |
| Organization Name |  |  | "Anturazh" Company Limited Liability Company |  |  |
| Address |  |  | 141420, Moscow region, Khimki, Skhodnya microdistrict, Nekrasov street, house 2, building 77, floor 4, part of room 23 |  |  |
| - Emergency telephone number |  |  | ## 8-499-394-02-60 |  |  |
| Fax |  |  | Absent |  |  |
| Е-mail |  |  | anturazh.co@mail.ru; info@anturazh.msk.ru |  |  |
| Web-site |  |  | Absent |  |  |
| - Hazards identification |  |  |  |  |  |
| # 2.1 Classification of the substance or mixture |  |  |  | Classification criteria according to Regulation (EC) 1272/2008: - serious eye damage/irritation, сategory 2A. - hazardous to the aquatic environment – ​​acute hazard, сategory 1. - hazardous to the aquatic environment – ​​chronic hazard, сategory 3. |  |
| # 2.2 Label Elements |  |  |  | Hazard pictograms: "Exclamation mark" "Environment" Signal word: Warning Hazard phrases: H319: Causes serious eye irritation. H400: Very toxic to aquatic life. H412: Harmful to aquatic life with long lasting effects. Precautionary phrases: P264: Wash hands thoroughly after handling. P273: Avoid release to the environment. P280: Wear eye protection. P305+P351+P338: IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P337+P313: If eye irritation persists, seek medical attention. P391: Collect spilled product. P501: Dispose of contents and container in accordance with local and national regulations. |  |
| # 2.3 Other hazards |  |  |  | This product is not classified as a PBT or vPvB substance and does not pose any other hazard according to Regulation (EC) No 1272/2008. |  |
| 3. Composition/information on ingredients |  |  |  |  |  |
| 3.1 Substances Not applicable 3.2 Mixtures |  |  |  |  |  |
| Component (INCI); Concentration (%); Number CAS; Number EC; Classification according to Regulation (EC) No 1272/2008 (CLP). Distilled water; 70; 7732-18-5; 231-791-2; Not classified. Isopropyl alcohol; 14,5; 67-63-0; 200-661-7; H225: Highly flammable liquid and vapour, Category 2. H319: Eye irritation, Category 2A. H336: Specific toxicity (single exposure), Category 3.. Alpha-alkyl C12-14-omega-hydroxypoly(oxy-1,2-ethanediyl) (nonionic surfactant); 15; 68439-50-9; 500-213-3; H400: Acute aquatic toxicity, Category 1. H411: Chronic aquatic toxicity, Category 2.. Benzyl acetate; 0,5; 140-11-4; 205-399-7; H412: Chronic aquatic toxicity, category 3.. |
| 4. First aid measures |  |  |  |  |  |
| 4.1 Description of first aid measures |  |  |  |  |  |
| In case of eye contact |  |  | Rinse eyes gently with running water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Seek medical attention if necessary. |  |  |
| In case of skin contact |  |  | Remove excess product with a cotton swab and rinse with soap and running water. If necessary, seek medical attention. |  |  |
| In case of ingestion |  |  | Drink plenty of water, take activated charcoal, and use a saline laxative. Seek medical attention if necessary.if necessary. |  |  |
| In case of inhalation |  |  | Fresh air, rest, warmth, and clean clothing. Seek medical attention if necessary. |  |  |
| 4.2 Most important symptoms and effects, both acute and delayed |  |  |  |  |  |
| Skin contact |  |  | Redness and dryness may occur. |  |  |
| Eye contact |  |  | Redness and tearing. |  |  |
| Ingestion |  |  | Weakness, headache, disturbances in respiratory rate and rhythm, nausea, vomiting, diarrhea. |  |  |
| Inhalation of high concentrations |  |  | Weakness, headache, disturbances in respiratory rate and rhythm, nausea, vomiting. |  |  |
| 4.3 Indication of any immediate medical attention and special treatment needed: None required |  |  |  |  |  |
| 5. Firefighting measures |  |  |  |  |  |
| 5.1 Extinguishing media |  |  |  |  |  |
| Suitable extinguishing agents |  |  |  | Foam, carbon dioxide, water spray, sand; indoor use – volumetric extinguishing. |  |
| Unsuitable extinguishing agents |  |  |  | It is not recommended to use water in the form of a compact stream, as this may result in the ejection or splashing of the burning product and intensification of the combustion. |  |
| 5.2 Special hazards arising from the substance or mixture |  |  |  |  |  |
| Hazardous combustion products |  |  |  | During combustion and thermal decomposition, toxic carbon oxides are formed. Carbon monoxide disrupts the transport and delivery of oxygen to tissues, leading to oxygen deficiency. Symptoms of poisoning include headache, dizziness, blurred vision, nausea, vomiting, and loss of consciousness. Carbon dioxide in a fire causes increased respiratory rate and pulmonary ventilation, and has a vasodilatory effect. Symptoms of poisoning include increased heart rate, increased blood pressure, migraine headaches, dizziness, lethargy, and loss of consciousness. |  |
| 5.3 Advice for firefighters |  |  |  |  |  |
| Wear suitable breathing apparatus. Collect contaminated firefighting water separately. Do not discharge it into the sewer system. Remove undamaged containers from the immediate hazard area if this can be done safely. |  |  |  |  |  |
| 6. Accidential release measures |  |  |  |  |  |
| 6.1 Personal precautions, protective equipment and emergency procedures |  |  |  |  |  |
| For non-emergency personnel |  |  |  | Observe fire safety precautions. Eliminate sources of ignition and sparks. Do not smoke. Wear protective clothing and personal protective equipment. |  |
| For emergency services |  |  |  | Stay upwind. Avoid low-lying areas. Wear personal protective equipment when entering the danger zone. Use explosion-proof electrical equipment and lighting when working. Provide first aid to injured persons. |  |
| 6.2 Environmental precautions |  |  |  |  |  |
| Prevent discharge into wastewater or groundwater. Collect contaminated rinse water and dispose of it properly. Notify the appropriate authorities if environmental damage occurs. |  |  |  |  |  |
| 6.3 Methods and materials for containment and cleaning up |  |  |  |  |  |
| Provide local ventilation. Collect in sealed containers and dispose of. |  |  |  |  |  |
| 7. Handling and storage |  |  |  |  |  |
| 7.1 Precautions for safe handling |  |  |  |  |  |
| During production, rooms must be equipped with supply and exhaust ventilation to ensure clean air in the work area. All work must be conducted away from fire and sparks. Do not drink, eat, or smoke while handling the product. Wash your hands thoroughly after handling. Maintain good personal hygiene. |  |  |  |  |  |
| 7.2 Conditions for safe storage, including any incompatibilities |  |  |  |  |  |
| The product should be stored in conditions protected from moisture and sunlight. Keep out of reach of children. Do not use at temperatures below 5°C or above 40°C. The warranty period is at least 24 months from the date of manufacture when stored in the original packaging in a cool, dry place. |  |  |  |  |  |
| 7.3 Specific end use(s) |  |  |  |  |  |
| Use only for its intended purpose. |  |  |  |  |  |
| 8. Exposure controls/personal protection |  |  |  |  |  |
| 8.1 Control parameters |  |  |  | Maximum permissible concentration of vapors in the air of the working area: Isopropyl alcohol – 50/10 mg/m3. Benzyl acetate – 5 mg/m3. |  |
| 8.2 Exposure controls |  |  |  |  |  |
| 8.2.1 Relevant engineering controls |  |  |  | Ensure adequate ventilation in the workplace. Under normal operating conditions, no special engineering controls are required. |  |
| 8.2.2 Individual protection measures, such as personal protective equipment: |  |  |  |  |  |
| Eye / face protection |  |  |  | If there is a risk of splashes, it is recommended to use safety glasses in accordance with EN 166. |  |
| Respiratory protection |  |  |  | Respiratory protection according to EN 149. |  |
| Hand protection |  |  |  | It is recommended to use protective gloves that comply with EN 374. |  |
| Skin and body protection |  |  |  | Protective workwear according to EN ISO 13688. |  |
| Thermal hazards |  |  |  | No thermal hazards are expected under normal conditions of use. |  |
| 8.2.3 Environmental exposure controls |  |  |  | Avoid uncontrolled release of the product into the environment. Under normal operating conditions, no special environmental impact controls are required. |  |
| 9. Physical and chemical properties |  |  |  |  |  |
| Specifications; Indicators. Appearance (physical state, color); A colorless, transparent liquid. Odor; With a slight odor. Hydrogen index, pH; 6,9 – 7,8. Freezing point, °C; -4. Boiling point, °C; 120. Flash point, °C; >250. Vapor pressure at 20 °C, Pa; <0,01. Density at 25 °C, g/cm3; 0,91. Viscosity at 25 °C, Pa*s; 8,90*10-4. Solubility; Soluble in water. 9.1 Information on basic physical and chemical properties: The information below relates to the basic physical and chemical properties of the product. 9.2 Other information: None |
| 10. Stability and reactivity |  |  |  |  |  |
| 10.1 Reactivity |  |  |  | The product is not reactive under normal conditions of storage, handling and use. |  |
| 10.2 Chemical stability |  |  |  | The product is chemically stable under normal conditions of storage, transportation and operation. |  |
| 10.3 Possibility of hazardous reactions |  |  |  | No hazardous reactions are expected under normal conditions of use. |  |
| 10.4 Conditions to avoid |  |  |  | Avoid excessive heat, open flames and ignition sources. |  |
| 10.5 Incompatible materials |  |  |  | No incompatible materials are known under normal conditions of use. |  |
| 10.6 Hazardous decomposition products |  |  |  | No hazardous decomposition products are expected under normal conditions. In case of fire, carbon oxides may be formed. |  |
| 11. Toxicological information |  |  |  |  |  |
| 11.1 Information on hazard classes as defined in Regulation (EC) No 1272/2008 |  |  |  |  |  |
| Acute toxicity |  |  | There is no data available for the product as a whole. Information is provided for individual components. Propan-2-ol: DL50 - 5840 mg/kg (intragastric, rats) DL50 - 12800 mg/kg (dermal, rabbits) CL50 - 37500 mg/m³ (inhalation, 4 hours, rats) alpha-AlkylC12-14-omega-hydroxypoly(oxy-1,2-ethanediyl): DL50 > 2000 mg/kg (intragastric, rats) Benzyl acetate: DL50 > 2000 mg/kg (intragastric, rats) DL50 > 5000 mg/kg (dermal, rabbits) CL50 - 766 mg/m³ (inhalation, 4 hours, rats) |  |  |
| Respiratory or skin sensitization |  |  | Sensitizing effect has not been established. |  |  |
| Mutagenicity |  |  | There is no data to indicate that the product or any components present at levels greater than 0.1% are mutagenic or genotoxic. |  |  |
| Carcinogenicity |  |  | None of the components of this product, when present in amounts greater than or equal to 0.1%, are classified by IARC as probable, possible, or confirmed human carcinogens. |  |  |
| Reproductive toxicity |  |  | The product does not have reproductive toxicity. |  |  |
| Specific target organ toxicity - single exposure |  |  | Not classified as a specific target organ toxicant. |  |  |
| Specific target organ toxicity - repeated exposure |  |  | Not classified as a specific target organ toxicant. |  |  |
| Aspiration hazard |  |  | Not classified as an aspiration hazard. |  |  |
| Chronic effects |  |  | No information |  |  |
| 11.2 Information on other hazards |  |  | No information |  |  |
| 12. Environmental information |  |  |  |  |  |
| 12.1 Toxicity |  |  |  | There is no data on the product as a whole. Information is provided by component. Propan-2-ol: CL50 - 9640 mg/l, fathead minnow, 96 h EC50 - 13299 mg/l, daphnia, 48 h IC50 > 1000 mg/l, freshwater algae, 72 h alpha-AlkylC12-14-omega-hydroxypoly(oxy-1,2-ethanediyl): CL50 - 0.423 mg/l, fish, 96 h EC50 - 0.125 mg/l, daphnia, 48 h ErC50 - 0.0446 mg/l, algae, 72 h Benzyl acetate: CL50 - 4 mg/l, fish, 96 h EC50 - 17 mg/l, daphnia, 48 h EC50 – 110 mg/l, algae, 72 h. NOEC – 52 mg/l, algae, 72 h. |  |
| 12.2 Persistence and degradability |  |  |  | There is no data on the product as a whole. Propan-2-ol is transformed in the environment to form acetone. |  |
| 12.3 Bioaccumulative potential |  |  |  | There is no data on the product as a whole. Propan-2-ol is not expected to accumulate in biological samples (log Pow ≤ 4). |  |
| 12.4 Mobility in soil |  |  |  | There is no relevant information available. |  |
| 12.5 Results of PBT and vPvB assessment |  |  |  | It is not a PBT or vPvB substance. |  |
| 12.6 Endocrine-disrupting properties |  |  |  | No other adverse environmental impacts (ozone depletion, endocrine disruptions, global warming potential) are expected from this product. |  |
| 12.7 Other adverse effects |  |  |  | No data. |  |
| 13. Disposal considerations |  |  |  |  |  |
|  |  |  |  | Production waste must be transferred for recycling or disposal to organizations licensed for waste management. All disposal activities must be carried out in accordance with established local and national regulations. |  |
| 14. Transport information |  |  |  |  |  |
| 14.1 Number UN |  |  |  |  | Not applicable. |
| 14.2 UN Proper shipping name |  |  |  |  | Shipping Name Absent. ## Transport Name: ## Soap composition "Seam Shine" for forming seams of epoxy grouts and sealants TM "DipoxSi Systems" ("Dipoxi Systems") |
| 14.3 Transport hazard class (es) |  |  |  |  |  |
| International Civil Aviation Organization/ International Air Transport Association (ICAO / IATA) |  |  |  |  | Not applicable. |
| Danger / Class / Division |  |  |  |  |  |
| Environmental hazards |  |  |  |  |  |
| Signs |  |  |  |  |  |
| International Maritime Dangerous Goods Code (IMDG)code |  |  |  |  | Not applicable. |
| Danger / Class / Division |  |  |  |  |  |
| Danger to the aquatic environment |  |  |  |  |  |
| Signs |  |  |  |  |  |
| The Intergovernmental Organisation for International Carriage by Rail(MKMPOG) /European agreement concerning the transport of dangerous goods (DOPOG) |  |  |  |  | Not applicable. |
| Danger / Class / Division. Signs. Code |  |  |  |  |
| 14.4 Packing group |  |  |  |  | Not assigned. |
| 14.5 Environmental hazards |  |  |  |  | Absent. |
| 14.6 Special precautions for user |  |  |  |  | Absent. |
| 14.7 Transport in bulk according to Annex II of MARPOL and the IBC Code |  |  |  |  | Not applicable. |
| 15. Regulatory information 15.1 Safety, health, and environmental regulations/legislation specific for the substance or mixture This Safety Data Sheet (SDS) is prepared in accordance with Regulation (EC) No. 1907/2006 (REACH). Classification and labeling are in accordance with Regulation (EC) No. 1272/2008 (CLP). HMIS НEALTH; 2.. FLAMMABILITY; 1.. PHYSICAL HAZARD; 0.. PERSONAL PROTECTION; I.. NFPA |
| 15.2 Chemical Safety Assessment A chemical safety assessment has not been conducted for this mixture. |  |  |  |  |  |
| 16. Additional information |  |  |  |  |  |
| 16.1 Abbreviations and Acronyms CAS: Chemical Abstracts Service EC: European Economic Community IARC: International Agency for Research on Cancer vPvB: Very Persistent, Bioaccumulative Toxic PBT: Persistent, Bioaccumulative Toxic ADR: European Agreement concerning the International Carriage of Dangerous Goods by Road (ADR) ICAO: International Civil Aviation Organization IATA: International Air Transport Association (IATA) IMDG: International Maritime Dangerous Goods (IMDG) IMO: International Maritime Organization (IMO) HMIS: Hazardous Materials Identification System NFPA: Standard Material Hazard Identification System for Emergency Response. |  |  |  |  |  |
| 16.2 Main data sources ECHA (European Chemicals Agency) GESTIS International Limit Values ​​Database |  |  |  |  |  |
| 16.3 Definition of the class of mixtures and the assessment method used Regulation (EC) No 1272/2008 [CLP]: The classification of the product has been carried out using a calculation method in accordance with CLP Regulation (EC) No 1272/2008 and available data on the ingredients. |  |  |  |  |  |
| 16.4 List of relevant H-phrases (number and explanation): H319: Causes serious eye irritation. H400: Very toxic to aquatic life. H412: Harmful to aquatic life with long lasting effects. |  |  |  |  |  |
| 16.5 List of relevant CLP classifications: Eye Irrit. 2: Chemicals that cause eye damage/irritation, Category 2A. Aquatic Acute 1: Chemicals that are acutely toxic to the aquatic environment, Category 1. Aquatic Chronic 3: Chemicals that are chronically toxic to the aquatic environment, Category 3. |  |  |  |  |  |
| 16.6 Training Tips: Read the Material Safety Data Sheet (MSDS) before using the product. |  |  |  |  |  |
| 16.7 Other information: |  |  |  |  |  |
| Gathered information: |  |  |  | The classification was based on a calculation method and expert opinion, taking into account the concentration of the component. |  |
| Actual date: |  |  |  | 12.02.2026 |  |
| Disclaimer: |  |  |  | The information provided in this Material Safety Data Sheet is based on data that is believed to be accurate at the date of preparation of this Material Safety Data Sheet. No liability is accepted for any damage or injury caused by abnormal use or due to non-compliance with recommended practices. The above information and the product are provided provided that the person receiving them must make their own determination as to the suitability of the product for their specific purpose and provided that they accept the risk of their use. In addition, no permission is granted or implied for the application of any patented invention without a license. The above information is believed to be accurate and reflect the information available to the manufacturer. However, this does not entail a guarantee for all the specific characteristics of the goods and does not serve as a basis for the emergence of contractual relations from a legal point of view. Current laws and regulations must be respected by the manufacturer’s successor under his responsibility. |  |
| General manager "Anturazh" LLC /Tyunkov S.V./ Print place |  |  |  |  |  |