| - IDENTIFICATION OF CHEMICAL PRODUCTS AND INFORMATION ON THE MANUFACTURER AND / OR SUPPLIER |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Technical name |  |  | ## Anti-slip treatment for floors made of porcelain stoneware, tiles, mosaics «GEKKON» ТМ «DipoxSi Systems» |  |  |
| Brief recommendations for use |  |  | Recommended for processing wetted tile surfaces in swimming pools and saunas, kitchens, showers and bathrooms, toilets, medical institutions, catering establishments, shopping and office centers. Surfaces can be processed both indoors and outdoors. |  |  |
| Name of the organization |  |  | Limited Liability Company "Anturazh" |  |  |
| Address |  |  | 141420, Russian Federation, Moscow region, Khimki, Skhodnya microdistrict, st. Nekrasova, house 2, building 77, floor 4, part of the room 23 |  |  |
| Emergency telephone number |  |  | ## +7-499-394-02-60 |  |  |
| Fax |  |  | Absent |  |  |
| Е-mail |  |  | anturazh.co@mail.ru info@anturazh.msk.ru |  |  |
| Web-site |  |  | Absent |  |  |
| - GHS HAZARD IDENTIFICATION |  |  |  |  |  |
| # Classification Warning marking |  |  |  | - chemical products causing skin corrosion/irritation, class 3 - chemical products causing serious eye damage/irritation, class 2B |  |
| # Signal word |  |  |  | Warning |  |
| # Hazard symbols |  |  |  | No symbol |  |
| # H-phrases # P-phrases |  | # Н |  | H316: Causes mild skin irritation. H320: Causes eye irritation. P264: Wash hands thoroughly after handling. P305+P351+P338: IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P332+P313: If skin irritation occurs: Get medical attention. P337+P313: If eye irritation persists: Get medical attention. |  |
| 3. COMPOSITION / INFORMATION ON INGREDIENTS |  |  |  |  |  |
| Component (INCI); Concentration (%); Number CAS; Number EC. Water; 98; 7732-18-5; 231-791-2. Orthophosphoric acid; 2; 7664-38-2; 231-633-2. |
| 4. FIRST AID MEASURES |  |  |  |  |  |
| In case of contact with eyes |  |  | Rinse with plenty of running water with the eye slit wide open for 15 minutes. Remove contact lenses if you use them and if it is possible to do so without difficulty. Seek medical attention with the product label. |  |  |
| When exposed to skin |  |  | Rinse with running water. Seek medical attention if necessary. |  |  |
| If swallowed |  |  | Rinse mouth with water, drink plenty of fluids, take activated charcoal. Seek medical attention if necessary. |  |  |
| Inhalation |  |  | Fresh air, rest, warmth. Seek medical attention if necessary. |  |  |
| Most important symptoms Potential Hazardous Effects |  |  | If inhaled: dizziness, headache, lethargy, sore throat, cough, difficulty breathing. In case of contact with the eyes: redness, lacrimation. When exposed to the skin: redness, dryness, itching. If swallowed: salivation, nausea, vomiting, abdominal pain, diarrhea. |  |  |
| 5. FIRE FIGHTING MEASURES |  |  |  |  |  |
| Flammability |  |  |  | Non-flammable substance. |  |
| Thermal degradation products |  |  |  | No information. |  |
| Fire and explosion safety |  |  |  | Fire and explosion hazard indicators are not achieved. |  |
| Extinguishing Methods |  |  |  | According to the main source of ignition. |  |
| Extinguishing Specificity |  |  |  | The combustion process may involve the product packaging, the combustion of which is accompanied by the release of toxic substances. Firefighter's combat clothing (jacket and trousers with removable heat-insulating linings) complete with a firefighter's rescue belt, mittens or gloves, a fire helmet, and special protective footwear. |  |
| 6. MEASURES IN CASE OF ACCIDENTAL RELEASE/LEAKAGE |  |  |  |  |  |
| Individual precautions |  |  |  | Isolate the danger zone within a radius of at least 50 m. Enter the accident zone in protective clothing and breathing apparatus. Remove bystanders. Observe fire safety measures. Do not smoke. Provide first aid to victims. |  |
| Environmental precautions |  |  |  | Maximum sealing of containers, communications and other equipment; periodic monitoring of the content of harmful substances in the air of the working area; cleaning the air of production facilities to established standards before discharging into the atmosphere. Do not allow products to enter water bodies, soil, sewage systems. |  |
| Contamination and cleaning |  |  |  | Eliminate the leak while observing safety precautions. Pump the contents into a serviceable container or into a drain container while observing the conditions for mixing liquids. In case of an intensive leak, enclose with an earthen embankment. Cover the spill site with sand, wash with plenty of water, embank and prevent the substance from getting into surface waters. Cut off the surface layer of soil with contamination, collect and remove for disposal while observing safety precautions. Cover the cut areas with a fresh layer of soil. Call in neutralization specialists. |  |
| 7. HANDLING AND STORAGE |  |  |  |  |  |
| Precautions and safe handling |  |  |  | The premises must be equipped with general exchange and local supply and exhaust ventilation, ensuring clean air in the working area. When working with the product, do not drink, eat, or smoke. Wash your hands thoroughly after work. Observe personal hygiene rules. |  |
| Precautionary measures Safe storage |  |  |  | Avoid contact with eyes. Do not inhale. Do not swallow. Do not apply to wounds or mucous membranes. The product is packaged in bottles, polypropylene canisters and placed in a cardboard box for transportation. The product is stored at a temperature of no lower than 5°C and no higher than plus 100°C in covered dry warehouses in the manufacturer's packaging. It is not allowed to store cleaning products under direct exposure to sunlight and near heating devices. The warranty period of the product is at least 24 months from the date of manufacture in unopened packaging of the manufacturer. |  |
| 8. INDIVIDUAL PROTECTION MEANS |  |  |  |  |  |
| Individual protection Equipment Control |  |  |  | During production, the premises must be equipped with general exchange and supply and exhaust ventilation, ensuring clean air in the working area. |  |
| Individual protection means |  |  |  | When working, it is necessary to use personal protective equipment: filter respirators, rubber gloves, safety glasses, work clothes. |  |
| Eye protection |  |  |  |  |  |
| Respiratory Protection |  |  |  |  |  |
| Hand skin protection |  |  |  |  |  |
| Workwear |  |  |  |  |  |
| Special tools |  |  |  | Аbsent |  |
| 9. PHYSICAL AND CHEMICAL PROPERTIES |  |  |  |  |  |
| Specifications; Indicators. Appearance; A colorless, slightly cloudy liquid. Odor; Odorless. рН; 4.7-5.5. Freezing point, °C; Minus 1. Initial boiling point, °C; 105. |
| 10. STABILITY AND REACTIVITY |  |  |  |  |  |
| Chemical stability |  |  |  | The product is stable when handling, storage and transportation conditions are met. |  |
| Solubility |  |  |  | Dissolves in water. |  |
| 11. TOXICOLOGICAL INFORMATION |  |  |  |  |  |
| Acute toxicity |  |  | Acute toxicity indices of orthophosphoric acid: LD50(oral, rats) = 2600 mg/kg LD50(dermal, rabbits) > 2000 mg/kg CL50(inhalation, 4 h, rats) = 15384 mg/m3 |  |  |
| Respiratory or skin sensitization |  |  | Sensitizing effect not established. |  |  |
| Mutagenicity |  |  | There is no data indicating that the product or any components present in an amount greater than 0.1% are mutagenic or genotoxic. |  |  |
| Carcinogenicity |  |  | None of the components of this product, present in amounts exceeding or equal to 0.1%, is classified by the IARC as a probable, possible or confirmed carcinogen for humans. |  |  |
| Reproductive toxicity |  |  | Does not have a negative effect on the reproductive function of the body. |  |  |
| Specific target organ toxicity - single exposure |  |  | It is not classified as a specific target organ toxicant. |  |  |
| Specific target organ toxicity - repeated exposure |  |  | It is not classified as a specific target organ toxicant. |  |  |
| Aspiration hazard |  |  | It is not classified as dangerous during aspiration. |  |  |
| Chronic effects |  |  | No information |  |  |
| 12. ENVIRONMENTAL INFORMATION |  |  |  |  |  |
| Aquatic microorganisms |  |  |  | Ecotoxicity indices of orthophosphoric acid: EC50 (daphnia, 48 h) > 100 mg/l IC50 (algae, 72 h) > 100 mg/l |  |
| Persistence and degradability |  |  |  | No information. |  |
| Bioaccumulation potential Soil mobility |  |  |  | No information. No information. |  |
| Other adverse effects |  |  |  | No other adverse environmental impacts (eg ozone depletion, endocrine disruption, global warming potential) are expected from the product. |  |
| 13. WASTE MANAGEMENT |  |  |  |  |  |
|  |  |  |  | Production waste is neutralized in accordance with sanitary norms and rules approved in accordance with the established procedure. Issues of recycling and disposal of waste products in the production process should be carried out on a contractual basis with a company licensed for waste disposal. |  |
| 14. TRANSPORT INFORMATION |  |  |  |  |  |
| 14.1 Number UN |  |  |  |  | Absent. |
| 14.2 Proper shipping and transport names |  |  |  |  | Shipping Name Absent. ## Transport Name: ## Anti-slip treatment for floors made of porcelain stoneware, tiles, mosaics «GEKKON» ТМ «DipoxSi Systems» |
| 14.3 Types of transport risks |  |  |  |  |  |
| International Civil Aviation Organization/ International Air Transport Association (ICAO / IATA) |  |  |  |  | Not applicable. |
| Danger / Class / Division |  |  |  |  |  |
| Environmental hazards |  |  |  |  |  |
| Signs |  |  |  |  |  |
| International Maritime Dangerous Goods Code (IMDG)code |  |  |  |  | Not applicable. |
| Danger / Class / Division |  |  |  |  |  |
| Danger to the aquatic environment |  |  |  |  |  |
| Signs |  |  |  |  |  |
| The Intergovernmental Organisation for International Carriage by Rail(MKMPOG) /European agreement concerning the transport of dangerous goods (DOPOG) |  |  |  |  | Not applicable. |
| Danger / Class / Division. Signs. Code |  |  |  |  |
| 14.4 Group packing |  |  |  |  | Not assigned. |
| 14.5 Environmental risks |  |  |  |  | Absent. |
| 14.6 Environmental risks with the app. II to the International Convention for the prevention of pollution from ships (MARPOL) 73/78 и code International Code for the Construction and Equipment of Ships, Carriers Dangerous Chemicals in Bulk (MRG) |  |  |  |  | It does not concern. |
| 14.7 Special precautions |  |  |  |  | Absent. |
| 15. REGULATORY INFORMATION HMIS НEALTH; 1.. FLAMMABILITY; 0.. PHYSICAL HAZARD; 0.. PERSONAL PROTECTION; I.. |
| NFPA REACH GHS MDGC ICAO/IATA |  |  |  |  |  |
| Аpplication II MARPOL 73/78 RID АDR |  |  |  |  |  |
| 16. ADDITIONAL INFORMATION |  |  |  |  |  |
| We believe that the information contained here is current as of the date of this Material Safety Data Sheet and is being offered in good faith. Since the use of this information and these opinions and the terms of use of the product are not under the control of LLC "Anturazh", it is the user's obligation to determine the conditions for the safe use of the product. |  |  |  |  |  |
| Gathered information: |  |  |  | LLC "Anturazh" |  |
| Actual date: |  |  |  | 17.06.2025 |  |
| Disclaimer: |  |  |  | The information provided in this Material Safety Data Sheet is based on data that is believed to be accurate at the date of preparation of this Material Safety Data Sheet. No liability is accepted for any damage or injury caused by abnormal use or due to non-compliance with recommended practices. The above information and the product are provided that the person receiving them must make their own determination as to the suitability of the product for their specific purpose and provided that they accept the risk of their use. In addition, no permission is granted or implied for the application of any patented invention without a license. The above information is believed to be accurate and reflect the information available to the manufacturer. However, this does not entail a guarantee for all the specific characteristics of the goods and does not serve as a basis for the emergence of contractual relations from a legal point of view. Current laws and regulations must be respected by the manufacturer’s successor under his responsibility. |  |
| General director LLC "Anturazh" / Tyunkov S.V. / Print place |  |  |  |  |  |